# PROJECT REPORT

## Smart Traffic Management System Using Arduino, RFID & IR Sensors

## 1. ABSTRACT

Traffic congestion and delayed emergency response are major challenges in urban environments. This project presents a Smart Traffic Management System that dynamically controls traffic signals based on real-time traffic density and provides priority passage to emergency vehicles using RFID technology. The system uses IR sensors to detect vehicle density and an RFID module to override traffic signals when authorized emergency vehicles are detected. Implemented on an Arduino UNO, the system operates in real time using a non-blocking finite state machine, ensuring high accuracy, responsiveness, and scalability for smart city applications.

## 2. INTRODUCTION

Traditional traffic lights operate on fixed timers, which leads to inefficiencies during low or high traffic conditions and delays emergency vehicles. With the rise of smart cities, there is a need for adaptive, intelligent, and low-cost traffic control systems.

This project proposes a cost-effective smart traffic signal system that:

- Adjusts signal timing automatically based on traffic density

- Prioritizes emergency vehicles

- Works in real time with minimal hardware

- Can be extended to multi-junction systems

## 3. PROBLEM STATEMENT

- Fixed-time traffic lights waste green time when no vehicles are present

- Emergency vehicles are delayed in traffic congestion

- Existing intelligent systems are expensive and complex

## 4. OBJECTIVES

- Design an adaptive traffic signal controller

- Detect traffic density using IR sensors

- Implement emergency vehicle priority using RFID

- Develop a non-blocking real-time control algorithm

- Ensure system scalability and reliability

## 5. SCOPE OF THE PROJECT

- Single-lane intersection (prototype level)

- Emergency vehicle identification via RFID

- Real-time traffic adjustment

- Offline operation (no internet dependency)

Future scope includes:

- Multi-way intersections

- Camera-based AI detection

- IoT cloud integration

- Government smart city deployment

## 6. SYSTEM REQUIREMENTS

### 6.1 Hardware Requirements

| Component | Quantity |
| --- | --- |
| Arduino UNO | 1 |
| IR Sensors | 2 |
| RFID RC522 | 1 |
| RFID Tags | 1–2 |
| LEDs (Red, Yellow, Green) | 3 |
| Resistors | 3 |
| Jumper Wires | As required |

### 6.2 Software Requirements

- Arduino IDE

- MFRC522 RFID Library

- Embedded C/C++

## 7. SYSTEM ARCHITECTURE

### 📊 Block Diagram (Explain in Viva)

IR Sensor 1 ─┐

├──> Arduino UNO ───> Traffic LEDs

IR Sensor 2 ─┘          │

└──> RFID Reader (Emergency Override)

### Description:

- IR sensors send traffic density data

- Arduino processes inputs using FSM logic

- RFID overrides traffic logic when emergency vehicle is detected

- LEDs act as traffic signals

## 8. WORKING METHODOLOGY

### Step-by-Step Operation

- System initializes in RED signal state

- IR sensors continuously monitor traffic

- Green time is calculated dynamically

- RFID is checked continuously (highest priority)

- Emergency vehicle → Immediate GREEN

- System returns to normal operation

## 9. FLOWCHART

START

↓

Initialize System

↓

Check RFID?

├─ YES → GREEN (Priority Mode)

└─ NO

↓

Read IR Sensors

↓

Calculate Traffic Density

↓

Set Green Time

↓

GREEN → YELLOW → RED

↓

LOOP

## 10. ALGORITHM

- Initialize I/O pins and RFID module

- Set default traffic signal to RED

- Continuously monitor RFID reader

- If emergency RFID detected:

  - Override signals

  - Set GREEN for fixed priority time

- Else:

  - Read IR sensor values

  - Determine traffic density

  - Allocate green signal duration

- Repeat loop continuously

## 11. IMPLEMENTATION DETAILS

### Key Techniques Used

- Finite State Machine (FSM)

- Non-blocking timing using millis()

- Interrupt-like priority handling

- Modular function-based design

This ensures:

- High responsiveness

- Accurate timing

- No system freezing

- Professional-grade logic

## 12. RESULTS & OUTPUT

- Successfully detects traffic density

- Dynamically adjusts signal timing

- Emergency vehicle gets instant clearance

- Stable and accurate real-time operation

## 13. ADVANTAGES

- Low cost

- Real-time operation

- Emergency-friendly

- Scalable architecture

- Smart city compatible

## 14. LIMITATIONS

- Prototype-level (single direction)

- IR sensors affected by environment

- RFID range is limited

## 15. FUTURE ENHANCEMENTS

- AI camera-based vehicle detection

- Multi-junction synchronization

- GSM/IoT monitoring

- Mobile app integration

- Government traffic analytics

## 16. APPLICATIONS

- Smart cities

- Hospitals & emergency routes

- School zones

- Industrial traffic control

- Defense and secure areas

## 17. CONCLUSION

The proposed Smart Traffic Management System effectively improves traffic flow and emergency response using adaptive control and priority-based signaling. The system demonstrates how embedded intelligence can significantly enhance urban traffic efficiency using low-cost hardware.

## 18. REFERENCES

- Arduino Official Documentation

- MFRC522 RFID Library

- IEEE Smart Traffic Systems Research Papers

- Smart City Traffic Management Studies

## 🔌 ARDUINO PIN CONFIGURATION

### LEDs

| Signal | Pin |
| --- | --- |
| Red | 8 |
| Yellow | 9 |
| Green | 10 |

### IR Sensors

| Sensor | Pin |
| --- | --- |
| IR1 | 2 |
| IR2 | 3 |

### RFID RC522

| RC522 | Arduino |
| --- | --- |
| SDA | 7 |
| SCK | 13 |
| MOSI | 11 |
| MISO | 12 |
| RST | 6 |
| 3.3V | 3.3V |
| GND | GND |

## 🎯 What makes it “smart” (not basic)?

Your system will:

- Detect traffic density using 2 IR sensors

- Adjust green light timing dynamically (more cars → longer green)

- Give priority using RFID (ambulance, police, VIP lane)

- Fallback to normal mode when no traffic is detected

This is competition-ready and interview-friendly.

## 🧠 System Logic (High Level)

### 1️⃣ Traffic Density Detection (IR Sensors)

- IR Sensor 1 (Near signal) → detects queue start

- IR Sensor 2 (Far from signal) → detects long queue

| IR1 | IR2 | Traffic Level | Green Time |
| --- | --- | --- | --- |
| 0 | 0 | No traffic | 10 sec |
| 1 | 0 | Low traffic | 20 sec |
| 1 | 1 | High traffic | 40 sec |

### 2️⃣ RFID Priority Override 🚑

If a registered RFID tag is detected:

- Immediately switch GREEN

- Hold green for fixed priority time

- Ignore IR sensors during this time

Use case:

- Ambulance

- Fire truck

- Government vehicle

### 3️⃣ LED Signal Control

You’ll simulate one junction:

- 🔴 Red

- 🟡 Yellow

- 🟢 Green

CODE:

#include <SPI.h>

#include <MFRC522.h>

/* ---------- PIN DEFINITIONS ---------- */

#define RED_LED     8

#define YELLOW_LED  9

#define GREEN_LED   10

#define IR1_PIN     2

#define IR2_PIN     3

#define SS_PIN      7

#define RST_PIN     6

MFRC522 rfid(SS_PIN, RST_PIN);

/* ---------- TIMING (ms) ---------- */

#define YELLOW_TIME   3000

#define RED_TIME      10000

#define RFID_GREEN    30000

#define GREEN_LOW     10000

#define GREEN_MEDIUM  20000

#define GREEN_HIGH    40000

/* ---------- STATE MACHINE ---------- */

enum TrafficState {

RED,

GREEN,

YELLOW,

RFID_MODE

};

TrafficState currentState = RED;

unsigned long stateStartTime = 0;

unsigned long greenDuration = GREEN_LOW;

/* ---------- EMERGENCY RFID UID ---------- */

byte emergencyUID[4] = {0x63, 0xF0, 0x0A, 0x0E};  // CHANGE THIS

/* ---------- FUNCTION DECLARATIONS ---------- */

void setRed();

void setYellow();

void setGreen();

bool checkRFID();

void updateTrafficDensity();

void setup() {

Serial.begin(9600);

pinMode(RED_LED, OUTPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(IR1_PIN, INPUT);

pinMode(IR2_PIN, INPUT);

SPI.begin();

rfid.PCD_Init();

setRed();

stateStartTime = millis();

Serial.println("Smart Traffic System Initialized");

}

void loop() {

/* ---------- RFID HAS HIGHEST PRIORITY ---------- */

if (checkRFID()) {

currentState = RFID_MODE;

stateStartTime = millis();

setGreen();

Serial.println("RFID PRIORITY MODE");

}

unsigned long currentMillis = millis();

switch (currentState) {

case RED:

if (currentMillis - stateStartTime >= RED_TIME) {

updateTrafficDensity();

currentState = GREEN;

stateStartTime = currentMillis;

setGreen();

}

break;

case GREEN:

if (currentMillis - stateStartTime >= greenDuration) {

currentState = YELLOW;

stateStartTime = currentMillis;

setYellow();

}

break;

case YELLOW:

if (currentMillis - stateStartTime >= YELLOW_TIME) {

currentState = RED;

stateStartTime = currentMillis;

setRed();

}

break;

case RFID_MODE:

if (currentMillis - stateStartTime >= RFID_GREEN) {

currentState = RED;

stateStartTime = currentMillis;

setRed();

Serial.println("RFID MODE ENDED");

}

break;

}

}

/* ---------- FUNCTIONS ---------- */

void updateTrafficDensity() {

int ir1 = digitalRead(IR1_PIN);

int ir2 = digitalRead(IR2_PIN);

if (ir1 == HIGH && ir2 == HIGH) {

greenDuration = GREEN_LOW;

Serial.println("Traffic: NONE");

}

else if (ir1 == HIGH && ir2 == LOW) {

greenDuration = GREEN_MEDIUM;

Serial.println("Traffic: LOW");

}

else if (ir1 == LOW && ir2 == HIGH) {

greenDuration = GREEN_MEDIUM;

Serial.println("Traffic: LOW");

}

else {

greenDuration = GREEN_HIGH;

Serial.println("Traffic: HIGH");

}

}

bool checkRFID() {

if (!rfid.PICC_IsNewCardPresent()) return false;

if (!rfid.PICC_ReadCardSerial()) return false;

for (byte i = 0; i < 4; i++) {

if (rfid.uid.uidByte[i] != emergencyUID[i]) {

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

return false;

}

}

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

return true;

}

/* ---------- LED CONTROL ---------- */

void setRed() {

digitalWrite(RED_LED, HIGH);

digitalWrite(YELLOW_LED, LOW);

digitalWrite(GREEN_LED, LOW);

}

void setYellow() {

digitalWrite(RED_LED, LOW);

digitalWrite(YELLOW_LED, HIGH);

digitalWrite(GREEN_LED, LOW);

}

void setGreen() {

digitalWrite(RED_LED, LOW);

digitalWrite(YELLOW_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

}

……………………………..